ZAVOS AQUA PARK RESORT

GENERAL SPECIFICATION

- LOADING BEARING FRAME

Reinforced concrete frame with slabs, beams and columns and foundations: pad footings or raft foundation according to specification.

- WALLING

- External walls are constructed of 200mm thick hollow bricks, and are externally rendered with cement mortar and internally plastered with gypsum-based plaster.

- Internal walls are constructed of 100 mm thick hollow bricks and are plastered with gypsum-based plaster.

- PLASTERING

All Internal surfaces are finished in three coats of plastering.  All external surfaces are of a Tyrolean finish.

- FLOORING

Living room – Dining room	   	Ceramic Tiles 		30cm x 60cm @ £12 CYP per metre2

Bedrooms       			Laminate parquet flooring

Kitchen				Ceramic Tiles   		30cm x 60cm @ £12 CYP per metre2

Corridors				Ceramic Tiles 		30cm x 60cm @ £12 CYP per metre2

Bathrooms				Ceramic Tiles 		30cm x 60cm @ £12 CYP per metre2

Verandas				Ceramic Tiles  		30cm x 60cm @ £12 CYP per Metre2

- DOORS AND WINDOWS

- The Entrance Door is made of solid wood construction.

- All external doors and windows are double-glazed and are made of aluminium.

- All internal doors are made of M.D.F finished with a laminate  finish.

- WARDROBES & CABINETS

Built-in sliding wardrobes  are made  with a melamine finish.

Italian Kitchen Cabinets and cupboards are made of M.D.F., finished externally with a Laminate finish, incorporating benchtops made of Melamine.

- WALL TILES

Kitchen				Ceramic Tiles   		£12 CYP per metre2

Bathrooms				Ceramic Tiles  		£12 CYP per metre2

- SANITARY WARE/FITTINGS

Sanitary ware is white and of deluxe quality and the fittings are chrome plated.

- BALUSTRADES/FENCING

- Veranda balustrades are constructed of metal and painted with a quality external paint.

- The front boundary wall is finished with a Tyrolean finish.

- All external walls are constructed of concrete and painted with a Tyrolean finish.

- PLUMBING & DRAINAGE

- Cold-water installation includes a 1m3 P.V.C. storage tank and the water pipes are made of P.V.C. Hot water installation pipes are constructed of P.V.C.  The system includes an electric boiler of 20 gallons capacity.

- All drainage pipes are constructed of P.V.C.

- ELECTRICAL INSTALLATION

Electrical installation includes an adequate number of lighting points, power points, T.V. points, telephone points and bell and cooker control.  There is provision for split-unit Airconditioning, includes electrical installation and copes piping.

- SWIMMING POOL

There is a “Communal Complex” two  Swimming Pools.

13.   TWO LIFTS

OPTIONAL FACILITIES

Solar Heating System

Pressurising Water System